# The Scenario of Arsenic in Drinking Water in Nepal

- Robin Lall Chitrakar1 and Amar Neku2

# Background/Introduction

Nepal is located south of Lhasa province of China and north of the states of Uttar Pradesh and Bihar in India. Administratively, it is divided into 5 regions, 14 zones, and 75 districts. It can be divided into three geographical regions namely, the mountains, foothills and terai on the basis of its topography and climate. The terai region is the southernmost belt having a width ranging from 30 km to 40 km and its altitude varies from 60 m to 310 m above mean sea level. The 20 districts of the terai region adjoining India are the plains forming the northern extension of the Indo-Gangetic Plain. Their geology is similar to the Bengal Delta Plain (BDP) in Bangladesh and West Bengal, India. Only in the late nineties, the problem of arsenic (As) contamination in the groundwater (shallow tubewells) of these terai districts of Nepal was identified. This problem has specially emerged as a critical environmental health issue in this SAARC (South Asia Association for Regional Cooperation) region.

The population of Nepal is about 23.4 million people according to the recent population census data (2001).  Of this, about 47% live in the 20 Terai districts and about 90% of these people use ground water for drinking and other purposes.  Although there is no firm data on the type and total numbers of tube wells, it is estimated that each Terai district has of the order of 10,000 shallow tube wells.  Of these, about 50% are installed by the government and other agencies with the remaining 50% installed by private companies and individuals.  In total therefore current estimates are that in Nepal there are about 200,000 tube wells providing drinking water for about 11 million people. In this context, contamination of Terai ground water with arsenic is an emerging issue for Nepal also.  Based on the arsenic tests carried out in the terai tube wells so far by different agencies working in this sector (Refer to Table 3), the rough estimated number of people in Nepal who might be using water with “high” arsenic is presented in Table 1 below.

Table 1:  Preliminary Estimate of Extent of Problem in Nepal

| Based on WHO guideline (10 g/litre) |  | Based on India and Bangladesh guideline (50 g/litre) |  |
| --- | --- | --- | --- |
| % Exposed | Population | % Exposed | Population |
| 29 | 3.19 million | 5 | 550,000 |

Notes:  This is a very preliminary estimate.  Therefore, use this summary with caution.

Arsenic- General Information

Arsenic (As) is a metallic main group element, found in Group V (b) of the Periodic Table with Atomic Number 33, and relative atomic weight 75. Arsenic is a natural part of the earth's crust in some parts of the globe and may be found in water, which has flowed through arsenic-rich rocks. There has been wide and varied debate about the source of the arsenic, but it is now widely accepted that the source of the problem in India and Bangladesh is generally from the underlying geological strata.  In Nepal - Arsenic is from naturally occurring sources since there is no evidence of drinking water being contaminated by industries. There are two main theories as to how this contamination occurs:

- Pyrite oxidation. In response to pumping or other natural water level variations, air or water with dissolved oxygen penetrates into the ground, leading to decomposition of sulphide minerals and the release of arsenic.

- Oxyhydroxide reduction. Arsenic is naturally transported and adsorbed onto fine-grained iron or manganese oxyhydroxides, which slowly break down under anaerobic conditions.

It is increasingly accepted that the second theory is the most likely explanation but the whole process is still not well understood.

Drinking arsenic-rich water over a long period is unsafe, and in some countries around the world the health effects are well known. The most commonly reported symptoms of chronic arsenic exposure; Arsenicosis may be put into three stages namely:

- Earlier Stage1: - Melanosis (Darkening or blackening of skin color. Small black spots or diffuse black patches particularly over the trunk and back then gradually over the whole body); Keratosis (Skin becomes hard and rough especially on Palms of hands and soles); Conjunctivitis (Reddening of eyes); Bronchitis (Infection of respiratory system); and Gastroenteritis (Includes nausea, vomiting, and loose motion) can be observed in this stage.

- Secondary Stage1: - Leuko–Melanosis (Black and white spots on various parts of the skin of the body); Hyperkeratosis (Rough nodules on palms and sole); Non-pitting edema (Swelling of leg); and Peripheral neuropathy (Terminal neurosis) can be observed in this stage. Even various complications of kidney and liver occur.

- Tertiary Stage1: - Gangrene (Necrosis with putrefaction of terminal limbs of the body); and Cancer (Cancer of skin, urinary bladder and lungs) can be observed in this stage. Eventually, failure of function of the liver and the kidney occur.

Studies show that high concentrations of arsenic in a community's well do not necessarily correlate with high levels of arsenic symptoms within the community. Studies also suggest that malnutrition, Hepatitis B, and smoking will accentuate the effects of arsenic poisoning and that arsenic concentrations in wells in close proximity may vary widely.

The Department of Water Supply and Sewerage (DWSS), as a leading government agency in the water sector of Nepal, has been involved in installing shallow tube wells in the terai region since its establishment in 1972 in order to safeguard drinking water from microbial contamination as well as to increase the coverage of access to safe water for more people to meet the set national goal as "Water for all in 2002". Later as the Bengal Delta Plain (BDP) in Bangladesh and West Bengal, India came up with this arsenic (chemical) contamination in ground water problem issue, the possibility of arsenic chemical contamination in some terai tube wells in Nepal was initially recognised only in the late 1990's. During that time, the two major actors in the arsenic sector have been the Department of Water Supply and Sewerage (with WHO and UNICEF support) and the Nepal Red Cross Society (NRCS) with the support of the Japanese Red Cross Society. Other water supply agencies namely, Rural Water Supply and Sanitation Sector Project (RWSSSP), Rural Water Supply and Sanitation Fund development Board (RWSSFDB), Nepal Water for Health (NEWAH) are now taking active steps in this area. Also, Nepal’s National Steering Committee on Arsenic (NSCA) has been formed and in operation for two years.  This steering committee consists of 14 members from different agencies involved in water and health sectors in Nepal and working in the arsenic related field with a coordinated approach. The Interim Arsenic Policy Document on Arsenic has been prepared by this NSCA, which has been approved (An interim guideline of a maximum 50 gL-1 arsenic in drinking water has been recommended) by the government in August 2000. Recently, DWSS with assistance from the UNICEF has initiated ‘Terai Tubewell Arsenic Testing and Mitigation Program’ with the objectives of (i) determining the extent of arsenic contamination in more tube wells in the Project area; (ii) identify patients with arsenicosis symptoms in the study area; (iii) make communities aware about arsenic; and (iv) determine the viability and effectiveness of different water supply options. The program has already conducted testing of 3896 tube wells by field test kits in 20 districts of the terai region for screening positive samples and has also conducted confirmation testing of 1772 tube well water samples in the laboratory by the Atomic Absorption Spectrophotometer with hydride generator (AAS-HG). More water samples will be analysed in the most affected terai region this year.

This paper deals with the outcomes of the tests and studies conducted till now by the DWSS especially focussing on ‘Terai Tube wells Arsenic Testing and Mitigation Program’ launched with assistance from UNICEF.

## Overall methodology

The following methods/activities were adopted during this program:

- UNICEF/Nepal provided Arsenic Field Test Kits developed by All India Institute of Health and Public Hygiene (AIIPHH) at Calcutta, India;

- These test kits were modified to make it more user friendly at the field level for the front line field technicians;

- Organised Arsenic Orientation Training to DWSS staff namely, Regional Directors, District Engineers, Assistant District Engineers, Overseers, and Water Supply and Sanitation Technicians (WSSTs);

- Supplied test kits together with required chemicals for field tests and logistics to the trained WSSTs working in 20 terai districts and also briefed all the trained WSSTs in their respective districts about the stepwise procedures to be undertaken in the field and at the office for field-testing and delivering the samples to the Laboratory for confirmation tests;

- Monitored all 20 districts during the field-testing period;

- Results collected together with all samples for further confirmation tests using the AAS-HG and also collected empty bottles used for microbiological presence/absence tests;

- Entry of all the data collected from 20 districts;

- Collected confirmation test results of all positive and also few negative samples from the field kit method from the laboratory using (AAS-HG);

- Analysed all test data which is under processing;

- More tests will be conducted in arsenic affected area;

- Information, Education, and Communication (IEC) materials are being developed;

- Orientation and community awareness programs will be launched using these IEC materials and available test data; and

- Safe options will be determined for the affected communities.

Field Testing

Field-testing kits developed by All India Health and Public Hygiene (AIIHPH) has been used to detect arsenic concentration greater than 0.05 mgL-1 (50 ppb or 50 μgL-1) in tube well drinking water. This kit has been recommended by the manufacturer to detect arsenic greater than 50 ppb (An interim guideline of a maximum 50 ppb or gL-1 arsenic in drinking water has been recommended in Nepal) in drinking water. The test procedures are based on the following principles:

- As (V) is reduced to As (III) by adding Potassium Iodide (KI) and Stannous Chloride (SnCl2) in the acidic environment;

- As (III) is allowed to react with the hydrogen ion, which would be generated by the reaction of hydrochloric acid (HCl) and Zinc granules to form arsine gas (AsH3);

- The arsine gas thus produced is passed through a column containing a roll of cotton moistened with lead acetate solution to avoid possible interferences due to sulfide and antimony present in the sample water; and

- This generated arsine produces a yellow-brown stain on mercuric bromide paper, which can be visibly detected.

Laboratory Testing

Having screened arsenic positive samples by the AIIHPH field kits, all positive as well as a few negative samples collected and labeled from the field have been tested in the laboratory using the Atomic Absorption Spectrophotometer with hydride generation (AAS-HG) This is a standard and a reliable method recommended in the Standard Method for the examination of arsenic analysis in water at the ppb concentration levels approved by the American Public Health Association and the USEPA. An MDL as low as 0.002 mgL-1 can be obtained in this method.

Results and Discussion

Studies show that test results may vary seasonally between pre-monsoon and post–monsoon period especially at tube wells located along the active flood plain area. In order to minimize the effect of this seasonal variation in our test results, all the tests were planned to be conducted during the pre-monsoon period, but in a few districts (Jhapa, Kanchanpur, and Banke) the monsoon had started during the test. However, there were only a very few tube wells located along the active flood plain since the location of tube wells were initially selected using the grid of 1.25 -2 Km2  (1 tube well in 1.25–2 Km2 grid) using the topographical maps having latitudes, longitudes, and altitudes. Moreover, only a few days rain may not increase the ground water table substantially in these districts to affect our test results significantly.

The field-test results of 3896 tube wells using AIIHPH test kit helped for screening positive samples. The positive samples and a few negative samples (Total 1772 #) were selected from the whole lot of preserved samples for confirmation tests in the laboratory by the Atomic Absorption Spectrophotometer with hydride generator (AAS-HG). The microbiological presence and absence tests for all (3896) tube wells were also conducted in order to observe if the tested tube wells were microbiologically contaminated.  These test results are presented in Table 2. The summary of arsenic occurrence in ground water of Nepal is also presented in Table 3, which is the collection of test data by different agencies involved in this sector.

# Table 2, Graphs 3, and 4 depicts that about 24 % (918 out of 3896) of tube wells were identified as arsenic positive from the AIIHPH field test kits. The AAS-HG has detected arsenic in 737 samples out of 918 positive samples. Only 98 numbers of tube wells have exceeded arsenic concentration greater than 0.05 mgL-1 and 281 numbers of samples have exceeded WHO Guideline Value of 0.01 mgL-1. Likewise, 880 out of 2978 negative samples were also tested using the AAS-HG. Surprisingly, 565 samples were detected with arsenic positive; 30 samples have exceeded 0.05 mgL-1 and 136 have exceeded the WHO Guideline Value of 0.01 mgL-1. Altogether 128 (3.3 %) samples were found to have exceeded Nepal’s Interim Arsenic Guideline Value of 0.05 mg L-1 and 10.7 % of samples have exceeded the WHO Guideline Value of 0.01 mgL-1. It shows that for the positive screening tests the AIIHPH field kits have provided the results in conservative side whereas a lot of undetected samples by the AIIHPH were detected positive by the AAS-HG. Therefore, it is too early to draw a conclusion at this point on the reliability of the AIIHPH since there are many factors that affects the sensitivity of the field-test namely, quantity of As. present in test water, strength of mercuric bromide paper, quantity and quality of other reagents, contact time, and also a person who perform tests. Looking at the control conditions in the laboratory, there is less likely of errors occurring in laboratory test results compared to the field situations. This is the reason of considering only laboratory results in preparing the summary data presented in Table 3. From the economy point of view test kits are feasible options for screening positive samples in Nepal’s village context, however, random testing of negative samples must also be tested using AAS-HG or other recommended laboratory methods for reconfirmation.

# Table 3 and Graph 1 demonstrates the occurrence of Arsenic in groundwater in different districts of Nepal as of November 26 2001.  Four districts namely, Rautahat, Nawalparasi, Parsa, and Bara seemed to have relatively more arsenic in groundwater than other districts. The table 3 data shows that 207, 188, 53, and 50 tube wells from the above districts, respectively contain more than Nepal’s Arsenic Interim Guideline value of 0.05 mgL-1. Altogether 5 % of tube wells have more than Nepal’s Arsenic Interim Guideline Value and 29 % of tube wells have more than the WHO Arsenic Guideline Value. These figures will further increase as more and more tests are still going on and only about 6.3 % of tube wells have been tested so far.

# Graph 2 provides us with the information that the shallow tube wells having arsenic concentration greater than 0.05 mgL-1 have the depth mostly concentrated at the range of 7 to 28 m. More testing and hydro-geological studies are required to establish a relation between the depth of tube wells versus arsenic concentrations.

# Table 2 shows that about 55 % of tested tube wells were found to be microbiologically contaminated indicating the need of improving sanitation around the tube well platform sites. Generally, platform are not properly constructed and maintained and sometimes not provided thereby leaking contaminated water. Also, contaminated water has been found to be used for priming tube wells.

# Future Action and Recommendations

Screening Tube wells by Blanket Testing

More testing (Blanket Testing) is needed to get more accurate small-scale understanding of the extent of arsenic in groundwater and arsenic free sources. Only 12500 out of 200,000 tube wells have so far been tested.

Vital Information Dissemination and Community Awareness Program

Communication with communities is very vital especially on the issues and ways to deal with the problem. Inform communities on the status of the tube wells and provide them with safe options. Demonstration of simple technologies is required to remove arsenic. Communicate the following basic messages with the help of IEC materials/Radio/TV/Village Meetings. (Developing and Printing of IEC materials are in the process).

- Do not use for drinking and cooking from the tube wells having arsenic greater than 0.05 mgL-1 (The spouts of these tube wells are painted white with black crosses).

- Drink from safe tube wells and allow others to share safe tube wells (Source substitution).

- Improve diet to include some protein and take vitamin supplements.

- Explain external symptoms of arsenicosis and refer to health centres.

- Demonstrate simple household arsenic removal technologies.

Providing Access to Safe Water

- Identification of Arsenic Safe existing shallow tube wells (unpainted tube wells); deep tube wells (studies show that deep tube wells are relatively safer than shallow tube wells).

- Safe Surface water.

- Rain Water Harvesting.

- Household/Community level arsenic removal technologies namely, 3-Kolsi (Pitcher or Gagri) Filter (relies on passive coagulation with iron and/or adsorption to sand matrix); Solar Oxidation and Removal of Arsenic (SORAS); filters using tablets; Activated Alumina/adsorption Filters; Passive sedimentation (Relies on passive coagulation with iron); DPHE/DANIDA Bucket Treatment Unit (Relies on enhanced coagulation by the use of Alum, less dependent upon ground water iron, chemical oxidant enhances arsenite, As (III), removal by the use of KMnO4). Stevens Institute of Technology (Relies on enhanced coagulation and co-precipitation using ferrous sulfate, less dependent upon groundwater iron, Chlorine based chemical oxidant enhances As (III) removal). It is to be emphasized that the drinking water supplied must not only be free from harmful levels of arsenic, but also from bacteriological contamination, and other chemical contaminants. Arsenic can not be removed from contaminated water by boiling or by using normal candle filter. These removal technologies can also effectively remove iron.

Provide Medical Care to the Affected

- Preparation of a trained network of health workers.

- For less affected patients: - stop drinking arsenic contaminated water, distribute vitamins A, B, C, and E, and provide micro-nutrients, proteins, and ointment.

- Referral for seriously affected patients.

Research Studies

- Hydro-geological and Geo-chemical study of sediments including ground water aquifers.

- Health research.

- Mitigation solutions.

Laboratory Facilities

- Regional laboratory facilities.

- Private tube well – testing and mitigation.

Needs of Institutionalisation

Only an administratively and financially secured institution can institutionalise the Arsenic Testing and Mitigation Program and a peace meal basis program will not be sustainable in the long run. The set up institution should conduct Arsenic-monitoring program to test groundwater at least twice during pre-monsoon and post-monsoon period since studies show that arsenic concentration in ground water varies seasonally. Also the institution should develop guidelines/regulations for installing a new tube well. These guidelines should make testing mandatory prior to installing a new tube well and also a tube well should not extract water from different aquifers to avoid possible inter-aquifer arsenic contamination if possible.

References:

- Elizabeth, M. Jones., (December 2000). An Overview of the Arsenic in Bangladesh.

- Jessica J. Hurd (June 2001). Evaluation of Three Arsenic Removal Technologies in Nepal. Submitted to the department of civil and environmental engineering in partial fulfilment of the requirements for the degree of Master Of Engineering in Civil and Environmental Engineering at the Massachusetts Institute Of Technology.

- Munir, A.K.M., Rasul, S.B., Habibuddowla, M., Alauddin, M. Hussam, A., and Khan, A.H. (May 2001). Evaluation of Performance of Sono 3-Kolshi Filter for Arsenic Removal from Groundwater Using Zero Valent Iron through Laboratory and Field Studies. In: Technologies for Arsenic Removal from Drinking Water, Eds. M. Feroz Ahmed, M. Ashraf Ali, and Zafar Adeel, p171-189Preprints of BUET – UNU International Workshop, Dhaka, Bangladesh.

- Nepal’s Interim Arsenic Policy Preparation Report (May 2001).

- Sharma, R.M., (1999). Research study on possible contamination of groundwater with Arsenic in Jhapa, Morang, and Sunsari districts of Eastern Terai of Nepal.

- Tandukar, N., Bhattacharya, P., and Mukherjee, A.B. (November 2001). Preliminary Assessment of Arsenic Contamination in Groundwater in Nepal.

1 Senior Divisional Engineer, Department of Water Supply and Sewerage, Nepal

2 M. Sc. in Environmental Engineering, Department of Water Supply and Sewerage, Nepal

1 An Overview of the Arsenic in Bangladesh. Water Aid, Bangladesh (2000).